MicroSmith, Version 2.000

Version 2.000 of MicroSmith represents a major update.  New

features have been added.  The program is now published and

distributed by ARRL.  A printed manual is included with the program

disc.

Program support is still done through Wes Hayward, 7700

S.W.Danielle Ave, Beaverton, OR 97005.  Please let me know if bugs

are discovered or if you have features that you would like to see

in later versions.

Have fun with MicroSmith.  Thanks for picking it for your RF

analysis!

Update Notes, Version 2.000A., 29 Nov 92 WH.

--------------------------------------------

1.  A last minute bug was found and fixed in the interpolation

routine used with the frequency dependent termination.  The program

changes are transparent to the user.

2.  A feature has been added in connection with the frequency

dependent termination function.  The user now has the option of two

different methods of interpolation.  These are accessed while

viewing the Smith Chart by pressing I or i.  Both interpolations

are linear and are applied to the reflection coefficients specified

as terminations.  One is a linear interpolation of the polar gamma

terminations while the other applies to the rectangular parameters.

The rectangular interpolation is the default case.  Try the switch

between modes to see which one best fits the specific problem that

you are studying.  In either case, keep in mind that the only place

where the calculations are accurate and meaningful will be at the

frequencies where the terminations were defined.

3.  In connection with the frequency dependent termination, a note

had been placed in the manual to warn the user to keep a frequency

sweep or "tuning" between the frequencies of termination

definition.  Code has now been added to automate this.  If you try

to tune F outside of the range, a short beep sounds and the

frequency is reset to be inside the limits.  If a sweep is

attempted outside the range of termination definition, the sweep

limits are reset to be within the range.

4.  The programs SC and SCTU have been modified to remind the user

to use the installation program, SCINST, if the program has not

been properly installed.

5.  Additional error trapping was added in the file-save function.

Update Notes, Version 2.000B., February 17, 1993 WH.

----------------------------------------------------

1.  A bug was found that caused a "lock-up" in an endless loop.

The problem occurred only in the multiple frequency termination

mode and was fixed with relative ease, once the problem was

encountered.  Many thanks to N6LF who discovered and reported it.

2.  The code for termination data entry has been expanded.

3.  The schematic editor section is accessed with F9.  You can now

return to the chart by pressing either F9 or ESC.

4.  An Applications note has been added to the end of this file.

Thanks again for using MicroSmith.  Don't forget to register your

copy with the ARRL using the card provided.

Version 2.001, April 29, 1993, WH.

1.  A minor bug was found and fixed in that part of the program that

interpolates between frequencies for the frequency dependent

termination.

2.  The program section for frequency dependent termination has been

expanded.  The earlier versions would only handle 8 frequencies.

MicroSmith will now deal with up to 100 frequencies.  The editor is

still limited:  It will display a table showing only the early

parameters, although it is still possible to edit and change any of the

terminations.  This change was made to accommodate wide band matching

problems with files created by other programs.  A file called

"TWOBAND.DAT" is now included on the distribution disc.  This file was

generated with ELNEC, version 3.01 and represents a wideband sweep of a

34 ft vertical monopole.  The file shows a matching network that

provides a simultaneous match at both 7 and 14 MHz.

3.  Some MicroSmith users may wish to write files with other programs

that will "feed" data into MicroSmith.  Data is included in the manual

for the *.GAM files.  The *.DAT files are more complicated.   The format

information is available if you want to write such files.  Drop me note

if you need this data.

4.  ELNEC users may encounter a problem in generating files with large

numbers of frequency points in the DAT files.  ELNEC version 3.00 will

write .DAT files with a maximum of 8 frequency steps.  To change the

number, an environment variable must be set.  This is done by entering

SET ENMSN=# where # is the maximum number of frequency steps in the .DAT

file.  (MicroSmith will accommodate #=100.)  The environment variable

must be set prior to running ELNEC and is effective until the computer

is turned off or rebooted.  It can be entered from the DOS command line

or placed in the AUTOEXEC.BAT file or in a batch file which can be used

to start ELNEC.  With ELNEC versions 3.01 and higher, the maximum number

of .DAT file frequency steps can be set with its setup program, ENSETUP.

Version 2.001A, July 1, 1993.

1.  A minor bug was found and fixed in the code related to a multiple

frequency sweep of a variable other than frequency.

MicroSmith Applications Note #2.0.

We got a letter from a user who encountered problems with a common

analysis that is done on a Smith Chart.  He knew the length of the

50 Ohm transmission line between his transmitter and the antenna

feed point.  He had measured the complex impedance at the

transmitter end of the line as 70-j25.  The problem is to now

calculate the terminal impedance at the antenna.

The solution is well know with a paper Smith Chart.  The measured

impedance is plotted on the chart after normalization.  Then, a

circle is drawn about the chart center, starting at the measured

impedance.  The motion about this circle continues through an angle

corresponding to the line length.  The direction of this motion is

critical; you move "toward the load."  If we had started with the

antenna Z as the impedance entered into the chart, the motion would

have been the opposite, "toward the generator."  These are the

labels on the charts from Analog Instruments, Inc.

The procedure is similar with MicroSmith.  Enter SC from DOS and

proceed to the schematic editor.  Press N to reset the system for a

"new" circuit.  Be sure to set up the frequency right away.  The

termination is then entered as a single frequency point of 70-j25.

In our example, the line length was 2.35 wavelength.  This

corresponds to 846 degrees.  Hence, Part A is entered as a 50 Ohm

transmission line.  The line length is entered as -846 degrees.

The negative sign corresponds to rotation "toward the load."

Pressing F9 (or ESC) from the schematic editor takes us back to the

chart where the resulting impedance is immediately observed as

30.87-j9.28.  Press G to note that the VSWR and reflection

coefficient magnitude do not change with line length.

There is another way to examine this problem with MicroSmith,

although the negative line length is probably the better method.

The alternative starts with another "new" schematic.  A load of

1+j0 is inserted.  Part A is set up as a series resistor of

variable (tuned) value while Part B is a tunable series reactance.

Part C is a transmission line with length +846 degrees.  After the

circuit is entered, the Mark function is set up to place a mark at

70-j25.

After the circuit is entered, we return to the chart for analysis.

We immediately press M to place the mark on the chart.  Parts A and

B are then tuned until the cursor is right over the marker.  This

happens when the variable resistor is 30 ohms and the reactance is

-9 ohms.  We add the 1 ohm termination to obtain the 31-j9 value

that represents our antenna feed point.

We urge you to run the program to try these experiments, perhaps

adapting them to some of your own data.